Assignment MT1-2019-4

Section 1

(Based on lecture notes)

Note: - Use the notations used in the class whenever necessary

Q.1.1) What are the ways to obtain mass transfer coefficient?

Q.1.2)  Derive an expression for mass transfer coefficient using following theories.

- Film theory

- Penetration theory

- Surface renewal theory

- Boundary layer theory

Q.1.3) Discuss how the flow of fluid associated with convective mass transfer is accounted for in the theories listed in Q.3

Q.1.4) Write the dimensionless numbers related to heat and mass transfer along with the physical significance of them

Q.1.5) Explain Reynolds’s analogy and Coulburn analogy.

Q.1.6) Explain different type of Mass Transfer Coefficient and develop the relationship among them.

Q.1.7) Provide the expressions for different type of Mass Transfer Coefficient for the following cases.

- A is diffusing through non-diffusing B.

- Equimolar counter diffusion.

Section-2

(Based on the worked out problems in the text book)

From book: Principle of Mass Transfer by B.K.Dutta

Ex.3.1  Page no.79

Ex.3.2  Page no.80

Ex.3.3  Page no.87

Ex.3.4  Page no.89

Ex.3.5  Page no.99

Ex.3.6  Page no.100

Ex.3.7 Page no.101

Ex.3.8 Page no.104

Ex.3.9  Page no.105

From book: Treybal

Ex. 3.4  page no.69

What is the heat-transfer analog to this Eq.

Section 3

Unsolved Problems

3.1)A stream of air at 100 kPa pressure and 300 K is flowing on the top surface of a thin flat sheet of solid naphthalene of length 0.2 m with a velocity of 20 m/sec.  The other data are:

Mass diffusivity of naphthalene vapor in air = 6 * 10–6 m 2/sec

Kinematic viscosity of air = 1.5 * 10–5 m 2.sc

Concentration of naphthalene at the air-solid naphthalene interface =1 * 10–5 kmol/m3

Calculate:

- the overage mass transfer coefficient over the flat plate

- the rate of loss of naphthalene from the surface per unit width

Note: For heat transfer over a flat plate, convective heat transfer coefficient for laminar flow can be calculated by the equation.

you may use analogy between mass and heat transfer

3.2)

3.3) A solid disc of benzoic acid 3 cm in diameter is spin at 20 rpm and 25°C.  Calculate the rate of dissolution in a large volume of water.  Diffusivity of benzoic acid in water is 1.0 * 10–5 cm2/sec, and solubility is 0.003 g/cc.  The following mass transfer correlation is applicable:

Sh = 0.62 Re ½  Sc 1/3

Where  and ω is the angular speed in radians/time.

3.4) Air flows over a solid slab of frozen carbon dioxide (dry ice) with an exposed cross-sectional surface area of 1*10-3 m2. The carbon dioxide sublimes into the 2 m/s flowing stream at a total release rate of 2.29 *10-4 mol/s. The air is at 293 K and 1.013* 105 Pa pressure. At that temperature, the diffusivity of carbon dioxide in air is 1.5 *10-5 m2/s and the kinematic viscosity of the air is 1.55*10-5 m2/s.

3.5) Determine the Schmidt number for methanol in air at 298 K and 1.013 *105 Pa and in liquid water at 298 K.

3.6) Dittus and Boelter proposed the following equation for correlating the heat-transfer coefficient for turbulent flow in a pipe

What should be the corresponding equation for the mass-transfer coefficient when the transfer is to a turbulent fluid flowing in a pipe?

3.7) Determine the Schmidt number for

(a) oxygen in air at 300 K and 1.0 atm; and

(b) oxygen in liquid water at 300 K.

At 300 K, the diffusion coefficient of oxygen in liquid water is

1.5*10-9 m2/s.